Product Specifications (Communication Specifications)

EFEM  with  Fortrend for dia. 150/200/300 mm wafers

(1st edition)

FORTREND CO., LTD

Revision History

| Version | Date | Revision Information | Person in Charge |
| --- | --- | --- | --- |
| 1.0 | 2023/2/24 | Newly created | FCD700 |

――――	Table of Contents	――――

Revision History	0

- Stage Number and System Configuration	1

- Communication Format	2

  - Communication parameter	2

  - Contents of communication format	2

  - Message "E/C => EFEM PC"	2

  - Message type	2

  - Base of message	3

  - Receive response	4

  - Abnormal termination report format	6

  - Event report format	6

  - Basic communication sequence	7

  - Communication error process	9

- Base Message	10

  - READY	10

  - INIT	10

  - ORGSH	11

  - LOCK	11

  - UNLOCK	12

  - DOCK	12

  - UNDOCK	13

  - OPEN	13

  - CLOSE	14

  - WAFSH	14

  - MAPDT	15

  - GOTO	16

  - LOAD	17

  - UNLOAD	18

  - TRANS	19

  - CHANGE	21

  - ALIGN	22

  - HOME	23

  - HOLD	23

  - RESTR	24

  - ABORT	24

  - EMS	25

  - ERROR	25

  - CLAMP	28

  - STATE	29

  - MODE	30

  - SIGOUT	32

  - SIGSTAT	33

  - EVENT	38

  - CSTID	39

- Parallel I/O Table (Option)	40

  - Input	40

  - Output	40

  - Parallel I/O timing chart	41

- SETUP Sequence	42

- E84 Automatic Transfer Sequence	43

  - Automatic carry-in sequence	44

  - Automatic carry-out sequence	44

- Pod Load Sequence	45

- Pod Unload Sequence	45

- Transfer Sequence	46

  - Pnxx→LLi (Common to single and dual blade)	46

  - UPPER LLA Pnxx (Common to single and dual-blade)	48

  - PnxxUPPER LLA, UPPER LLAPnxx wafer exchange (only for dual-blade)	49

- Error Recovery Sequence	50

  - In case of finger clamp on error	50

  - In Case of detection of abnormal wafer during mapping	50

  - In case of other errors	51

- Receive Response When Canceling Message (CAN)	52

  - List of receive response when canceling message per message	52

  - Receive response when cancelling message	54

- Error Message (ABS)	56

  - Error code list for each message	56

  - Error recovery method	59

- Stage Number and System Configuration

Identification name of each stage is as follows:

TCP/IP

Parallel I/O

TCP/IP

Load port 4

[P4]

Load port 3

[P3]

Load port 2

[P2]

Load port 1

[P1]

Aligner

Robot

HUB

Port on system side [LLLB]

Port on system side [LLLA]

EFEM PC

E/C

Port on system side [ULLB]

Port on system side [ULLA]

On the Equipment side

On the EFEM side

Configuration of the EFEM

- Robot	: Fortrend Robot(Dual-finger)

- Aligner	: (Optional)

- Load port	: PLM150W/PLM200W/OCS/LoadPort

Pod slot numbers are as follows. Regardless of number of slots, the bottom slot is No.1.

No.25

No.24

No.23

No.22

· · ·

No.4

No.3

No.2

No.1

<=	Very top slot

<=	Very bottom slot

- Communication Format

  - Communication parameter

EFEM PC is a server by TCP/IP and connection is performed using desired port No. specified by the setting file. (Default port No. is "13000".)

  - Contents of communication format

    - MSG	Indicates the message or response.

    - DEL	Indicates the end of the message.

- CR: ASCII "0Dh"

In case of origin search

| M | O | V | : | O | R | G | S | H | ; | DEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Message "E/C => EFEM PC"

DEL

MSG

The format of the MSG part is as follows:

| Message type | : | Message base | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

There are multiple parameters or no parameter according to the message. Also, there are some messages which add the data to parameter.

Each item of message is delimited by ":", "/", ">", "|", and ";".

Meaning of each symbol

":" (ASCII "3Ah"):	Used between message type and message base.

"/" (ASCII "2Fh"):	Used between message base and parameter, or between parameter and parameter. ">" (ASCII "3Eh"): Used between parameter and parameter.

"|" (ASCII "7Ch"):	Used for the connection of error contents when error occurs. ";" (ASCII "3Bh"): Used at the end of the message.

  - Message type

    - For primary message

- Message which is issued by E/C

| Message type | Descriptions | Communication direction | Remarks |
| --- | --- | --- | --- |
| MOV | Motion request | E/C=>EFEM PC |  |
| GET | Request for each type status acquisition | E/C=>EFEM PC |  |
| SET | Control setting | E/C=>EFEM PC |  |

- Report of completion or information/event for the messages issued by E/C

| Message type | Descriptions | Communication direction | Remarks |
| --- | --- | --- | --- |
| INF | Information report | E/C<=EFEM PC |  |
| ABS | Abnormal termination | E/C<=EFEM PC |  |
| EVT | Event report | E/C<=EFEM PC | It can be disabled by EFEM setting. |

    - For secondary message

| Message type | Descriptions | Communication direction | Remarks |
| --- | --- | --- | --- |
| ACK | Normal receive | E/C< = >EFEM PC | Response to the message except ACK, NAK, CAN, and EVT |
| NAK | Communication error | E/C<=EFEM PC | Same as above (at the time of error) |
| CAN | Message cancel | E/C<=EFEM PC | Same as above (at the time of cancel) |

  - Base of message

| Base of message | Function | Message type which can use the corresponding message |
| --- | --- | --- |
| READY | Ready | , ACK, INF |
| INIT | EFEM initialization | MOV, ACK, INF |
| ORGSH | Origin search | MOV, ACK, INF |
| LOCK | Pod lock | MOV, ACK, INF |
| UNLOCK | Pod unlock | MOV, ACK, INF |
| OPEN | Pod open | MOV, ACK, INF |
| CLOSE | Pod close | MOV, ACK, INF |
| WAFSH | Wafer mapping | MOV, ACK, INF |
| MAPDT | Wafer map information | GET, ACK, INF, EVT |
| GOTO | Move to specified object | MOV, ACK, INF |
| LOAD | Wafer carry-out | MOV, ACK, INF |
| UNLOAD | Wafer carry-in | MOV, ACK, INF |
| TRANS | Wafer transfer | MOV, ACK, INF |
| CHANGE | Wafer exchange | MOV, ACK, INF |
| ALIGN | Alignment | MOV, SET, ACK, INF |
| HOME | Home | MOV, ACK, INF |
| HOLD | Hold | MOV, ACK, INF |
| RESTR | Restart | MOV, ACK |
| ABORT | Abort termination | MOV, ACK, INF |
| ERROR | Error | GET, SET, ACK, INF |
| CLAMP | Clamp output, get state | GET, SET, ACK, INF |
| STATE | Get status | GET, ACK, INF |
| MODE | E84 mode setting | GET, SET, ACK, INF |
| SIGOUT | Signal output | SET, ACK |
| SIGSTAT | Signal input/output information | GET, ACK, INF, EVT |
| CSTID | Carrier ID | GET, SET, ACK, INF |

  - Receive response

Send the response to the primary message.

    - Normal receive response

In case of response to the message (Message type = MOV, GET, SET) sent by E/C, this shows that the message is properly received and started processing.

In case of response to the message (Message type = INF, ABS) sent by EFEM PC, this shows that the message is properly received.

| ACK | : | Message＊ | ; |
| --- | --- | --- | --- |

*The contents between ":" and ";" of the received primary message are stored.

    - Error receive response

This shows that the message could not be properly received to the message that E/C sent.

| NAK | : | Factor | ¦ | Message＊ | ; |
| --- | --- | --- | --- | --- | --- |

*The contents between ":" and ";" of the received primary message are stored. (2-1) Factor

| Factor | Descriptions | Remarks |
| --- | --- | --- |
| MSG | Message error | Received undefined message. |

    - Receive response when cancelling message

This shows that the requested process cannot be performed to the message (Message type = MOV, GET, SET) sent by E/C.

| CAN | : | Message＊ | ¦ | Factor | / | Place | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

*The contents between ":" and ";" of the received primary message are stored. (3-1) Factor

Refer to "11.2 Receive response when cancelling message" for details.

| Factor | Descriptions | Place |
| --- | --- | --- |
| NOREADY | Communication not established | N/A |
| HOLD | Holding | N/A |

  - Abnormal termination report format

Symbol ‘|’ and error information which caused error are added to the message which ended abnormally. (Reference --- Base message: ERROR)

| ABS | : | Message* | ¦ | ERROR | / | Parameter1 | / | Parameter2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

*The contents between ":" and ";" of the received primary message are stored. ACK to ABS returns all contents between ":" and ";" which are sent with ABS.

  - Event report format

Supplementary information such as I/O change is sent as event information.

| EVT | : | Message* | ; |
| --- | --- | --- | --- |

*Base message and parameter which support EVT message are stored. (Reference --- Base message: MAPDT, TRANSREQ, SIGSTAT)

*It can be disabled by the system setting. In this case, acquire information by information acquisition message as needed.

  - Basic communication sequence

- Control sequence

This is used when E/C instructs the operation such as load/unload to EFEM. MOV =>	: Control command

<= ACK	: Normal received response of control command

<= INF	: Completion report of control command ACK =>	: Response of report

- Control sequence (Cancel of received contents)

This is used when it is judged that the EFEM cannot perform the operation though wafer transfer etc. is instructed from E/C to EFEM.

MOV =>	: Control command

<= CAN	: Cancel of control command

**** Wait until the timer which E/C side set is cleared or the operation is completed at EFEM side. **** MOV =>	: Control command (retry)

- Control sequence (Receive error)

This is used when the information from EFEM competes though E/C instructed wafer transfer etc. to EFEM. MOV =>	: Control command

<= NAK	: Control command error

*****	Resend after the timer set at E/C side	***** MOV =>		: Control command (retry)

- Control sequence (Unrecoverable error occurrence)

This is used when the error occurs inside EFEM during loading, etc. and the operation is aborted.

MOV =>	: Control command

<= ACK	: Normal received response of control command

*****	Error occurs inside EFEM	*****

<= ABS	: Abnormal completion report of control command ACK =>	: Response of report

- Control sequence (Recoverable error occurrence)

This is used when the error occurs inside EFEM during loading, etc. and the operation is aborted.

MOV =>	: Control command

<= ACK	: Normal received response of control command

*****	Error occurs inside EFEM	*****

<= ABS	: Abnormal completion report of control command ACK =>	: Response of report

*****	After handling of error	***** MOV =>		: Control command (retry)

<= ACK	: Normal received response of control command (retry)

<= INF	: Completion report of control command (retry) ACK =>	: Response of report

- Hold sequence

MOV =>	: Hold (HOLD message)

<= ACK	: Normal received response of hold

<= INF	: Hold completion information ACK =>	: Response of report

*****	In case of restart	*****

MOV =>	: Restart (RESTR message)

<= ACK	: Normal received response of restart

*****	In case of abort	*****

MOV =>	: Abort (ABORT message)

<= ACK	: Normal received response of abort

<= INF	: Abort completion information ACK =>	: Response of report

- Setting sequence

This is used when each kind of setting is done from E/C to EFEM. SET =>	: Setting

<= ACK	: Normal received response of setting

- Get state sequence

This is used when each kind of state for transfer is inquired from E/C to EFEM. GET =>	: Request for getting state

<= ACK	: Normal received response of get state request

<= INF	: State report (Some need more time to get state, so ACK and INF are reported separately.)

ACK =>	: Response of report

- Information sequence

This is used to notify the state change of control system such as pod carry-in/carry-out or error occurrence/release from EFEM to E/C.

<= INF	: Information

ACK =>	: Response of information

- Event sequence

This is used to notify supplementary information such as the change of input and output signals from EFEM to E/C.

<= EVT	: Event

  - Communication error process

- In case of checksum error

(Cause)	Message is not sent properly due to communication error (noise etc.). (Action)	Send the message again (Retry).

- In case that "ACK" is garbled

(E/C side)	Send the message again (Retry). (EFEM side) The message is sent again. (Retry)

- In case "ACK" is not returned within the specified time frame (five seconds) (E/C side)	Send the message again (Retry).

(EFEM side) The message is sent again (Retry). (max. three times)

- In case that one message cannot be received in the specified time frame (three seconds)

(E/C side)	Delete the message which is being received. Since "ACK" is not returned to EFEM side, message is resent (retried) from EFEM side.

(EFEM side) Message which is being received is deleted. Since "ACK" is not returned to E/C side, resend (retry) message from E/C side.

- Base Message

  - READY

- Function: Reports READY information.

- Report of READY (E/C <= EFEM PC)

| INF | : | READY | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Response of READY (E/C => EFEM PC)

| ACK | : | READY | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- READY parameter Parameter cannot be omitted.

| Parameter | Descriptions | Remarks |
| --- | --- | --- |
| COMM* | Communication READY | Establishment of communication between E/C and EFEM PC |

*After EFEM is started, EFEM sends this message periodically until "ACK" is returned from E/C. EFEM PC sets the periodical timer.

  - INIT

- Function: Initializes each unit of EFEM.

- Request for INIT (E/C => EFEM PC)

| MOV | : | INIT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of INIT (E/C <= EFEM PC)

| ACK | : | INIT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of INIT (E/C <= EFEM PC)

| INF | : | INIT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion response of INIT (E/C => EFEM PC)

| ACK | : | INIT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Parameter of INIT Parameter can be omitted.

If the parameter is omitted, the system acts in the same manner as when "ALL" is designated.

| Parameter | Descriptions | Remarks |
| --- | --- | --- |
| ALL | Designates whole EFEM. |  |
| ROB | Designates robot. |  |
| ALIGN | Designates aligner. |  |
| Pn | Designates load port n. | n is port number. (1 to) |

  - ORGSH

- Function: Performs origin search for EFEM.

- Request for ORGSH (E/C => EFEM PC)

| MOV | : | ORGSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of ORGSH (E/C <= EFEM PC)

| ACK | : | ORGSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of ORGSH (E/C <= EFEM PC)

| INF | : | ORGSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion response of ORGSH (E/C => EFEM PC)

| ACK | : | ORGSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Parameter of ORGSH Parameter can be omitted.

If the parameter is omitted, the system acts in the same manner as when "ALL" is designated.

| Parameter | Descriptions | Remarks |
| --- | --- | --- |
| ALL | Designates whole EFEM. |  |
| ROB | Designates robot. |  |
| ALIGN | Designates aligner. |  |
| Pn | Designates load port n. | n is port number. (1 to) |

  - LOCK

- Function: Locks pod clamp.

- Request for LOCK (E/C => EFEM PC)

| MOV | : | LOCK | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of LOCK (E/C <= EFEM PC)

| ACK | : | LOCK | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of LOCK (E/C <= EFEM PC)

| INF | : | LOCK | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of LOCK (E/C => EFEM PC)

| ACK | : | LOCK | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of LOCK

Designates the target port for motion.

| Parameter 1 | Contents | Remarks |
| --- | --- | --- |
| Pn | Designates load port n. | n is port number. (1 to) |

- Parameter 2 of LOCK (can be omitted)

Designates whether or not the carrier ID reading is performed.

| Parameter 2 | Contents | Remarks |
| --- | --- | --- |
| 0 | Carrier ID reading not performed | In the case the parameter is omitted |
| 1 | Carrier ID reading performed | Acquire the result with EVT:CSTID or GET:CSTID. |

  - UNLOCK

- Function:Unlocks pod clamp.

If pod is not closed, closes pod.

If pod is not moved to undock position, unlocks pod clamp after moving pod to undock position.(just for LP300)

- Request for UNLOCK (E/C => EFEM PC)

| MOV | : | UNLOCK | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of UNLOCK (E/C <= EFEM PC)

| ACK | : | UNLOCK | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of UNLOCK (E/C <= EFEM PC)

| INF | : | UNLOCK | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion response of UNLOCK (E/C => EFEM PC)

| ACK | : | UNLOCK | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Parameter of UNLOCK

Designates the target port for motion.

| Parameter | Contents | Remarks |
| --- | --- | --- |
| Pn | Designates load port n. | n is port number. (1 to) |

  - OPEN

-Function: Opens pod and performs wafer mapping at the same time.

If pod clamp is not locked, pod clamp is locked.

If pod is not moved to dock position, the pod is opened after moving it to dock position.(just for LP300)

- Request for OPEN (E/C => EFEM PC)

| MOV | : | OPEN | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of OPEN (E/C <= EFEM PC)

| ACK | : | OPEN | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of OPEN (E/C <= EFEM PC)

| INF | : | OPEN | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of OPEN (E/C => EFEM PC)

| ACK | : | OPEN | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of OPEN

Designates the target port for motion.

| Parameter 1 | Contents | Remarks |
| --- | --- | --- |
| Pn | Designates load port n. | n is port number. (1 to) |

- Parameter 2 of OPEN (can be omitted)

Designates whether or not the carrier ID reading is performed.

| Parameter 2 | Contents | Remarks |
| --- | --- | --- |
| 0 | Carrier ID reading not performed | In the case the parameter is omitted |
| 1 | Carrier ID reading performed | Acquire the result with EVT:CSTID or GET:CSTID. |

  - CLOSE

-Function: Closes pod but not performs unlock

- Request for CLOSE (E/C => EFEM PC)

| MOV | : | CLOSE | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of CLOSE (E/C <= EFEM PC)

| ACK | : | CLOSE | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of CLOSE (E/C <= EFEM PC)

| INF | : | CLOSE | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion response of CLOSE (E/C => EFEM PC)

| ACK | : | CLOSE | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Parameter of CLOSE

Designates the target port for motion.

| Parameter | Descriptions | Remarks |
| --- | --- | --- |
| Pn | Designate load port n. | n is port number. (1 to) |

  - WAFSH

-Function: Performs wafer mapping for opened pod.

If pod is closed, open the box first.

This function just for LP300

- Request for WAFSH (E/C => EFEM PC)

| MOV | : | WAFSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of WAFSH (E/C <= EFEM PC)

| ACK | : | WAFSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of WAFSH (E/C <= EFEM PC)

| INF | : | WAFSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion response of WAFSH (E/C => EFEM PC)

| ACK | : | WAFSH | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Parameter of WAFSH

Designates the target port for wafer mapping.

| Parameter | Descriptions | Remarks |
| --- | --- | --- |
| Pn | Designates load port n. | n is port number. (1 to) |

  - MAPDT

-Function: Acquires wafer state inside pod.

- Request for MAPDT (E/C => EFEM PC)

| GET | : | MAPDT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of MAPDT (E/C <= EFEM PC)

| ACK | : | MAPDT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of MAPDT (E/C <= EFEM PC)

| INF | : | MAPDT | / | Parameter | / | Data | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of MAPDT (E/C => EFEM PC)

| ACK | : | MAPDT | / | Parameter | / | Data | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Event response of MAPDT (E/C <= EFEM PC)

If the mapping result event is valid, this is sent after motions of OPEN and WAFSH of the load port are completed.

| EVT | : | MAPDT | / | Parameter | / | Data | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter of MAPDT Parameter cannot be omitted.

| Parameter | Descriptions | Remarks |
| --- | --- | --- |
| Pn | Designates load port n. | n is port number. (1 to) |

- Data of MAPDT

Reports wafer presence for number of slots.

| Data | Descriptions | Remarks |
| --- | --- | --- |
| 0 1 2 3 4 7 8 9 | No wafer in slot Normal wafer in slot Abnormal-thickness wafer in slot (Thick wafer) Cross-slotted wafer in slot Bow wafer in slot (Warped wafer) Multiple wafers in slot (two or more wafers in the same slot) Abnormal-thickness wafer in slot (Thin wafer) Others | Very bottom slot (left) to very top slot (right) |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

GOTO

-Function: Moves the robot to the front of the designated object.

- Request for GOTO (E/C => EFEM PC)

| MOV | : | GOTO | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of GOTO (E/C <= EFEM PC)

| ACK | : | GOTO | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of GOTO (E/C <= EFEM PC)

| INF | : | GOTO | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of GOTO (E/C => EFEM PC)

| ACK | : | GOTO | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of GOTO Designates the moving position.

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| Pnxx | Designates load port and slot. | n is port number. (1 to), xx is slot number. (01 to) |
| ALIGN | Designates aligner. |  |
| LLA | Designates LLA. | Slot 1 is targeted. |
| LLAxx | Designates LLA and slot. | xx is slot number. (01 to) |
| LLB | Designates LLB. | Slot 1 is targeted. |
| LLBxx | Designates LLB and slot. | xx is slot number. (01 to) |

- Parameter 2 of GOTO

Designates the finger to access the moving position.

| Parameter 2 | Descriptions | Remarks |
| --- | --- | --- |
| ARM1 | Designates upper finger. |  |
| ARM2 | Designates lower finger. |  |

Note: The single arm robot does not have lower finger. Therefore, if "ARM2" is designated, error message "NAK:MSG" is returned.

- Parameter 3 of GOTO

Designates the height at moving position for inserting a wafer or designates the height for picking up a wafer.

| Parameter 3 | Descriptions |
| --- | --- |
| UP | Height for inserting a wafer |
| DOWN | Height for picking up a wafer |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

LOAD

-Function: The robot carries out a wafer from designated position.

- Request for LOAD (E/C => EFEM PC)

| MOV | : | LOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of LOAD (E/C <= EFEM PC)

| ACK | : | LOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of LOAD (E/C <= EFEM PC)

| INF | : | LOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of LOAD (E/C => EFEM PC)

| ACK | : | LOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of LOAD

Designates the position of the wafer which is carried out.

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| Pnxx | Designates load port and slot. | n is port number. (1 to), xx is slot number. (01 to) |
| ALIGN | Designates aligner. | Alignment is not performed automatically before carry-out. Send "MOV:ALIGN;", if necessary. |
| ALIGN2 | Designates aligner. | Alignment is performed automatically before carry-out. |
| LLA | Designates LLA. | Slot 1 is targeted. |
| LLAxx | Designates LLA and slot. | xx is slot number. (01 to) |
| LLB | Designates LLB. | Slot 1 is targeted. |
| LLBxx | Designates LLB and slot. | xx is slot number. (01 to) |

- Parameter 2 of LOAD

Designates the finger which is used for carrying out.

| Parameter 2 | Descriptions | Remarks |
| --- | --- | --- |
| ARM1 | Designates upper finger. | Upper finger carries out a wafer. |
| ARM2 | Designates lower finger. | Lower finger carries out a wafer. |

Note: The single arm robot does not have lower finger. Therefore, if "ARM2" is designated, error message "NAK:MSG" is returned.

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

UNLOAD

-Function: The robot carries a wafer in designated position.

- Request for UNLOAD (E/C => EFEM PC)

| MOV | : | UNLOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of UNLOAD (E/C <= EFEM PC)

| ACK | : | UNLOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of UNLOAD (E/C <= EFEM PC)

| INF | : | UNLOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of UNLOAD (E/C => EFEM PC)

| ACK | : | UNLOAD | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of UNLOAD

Designates the position of the wafer which is carried in.

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| Pnxx | Designates load port and slot. | n is port number. (1 to), xx is slot number. (01 to) |
| ALIGN | Designates aligner. | Alignment is performed automatically after carry-in. |
| ALIGN2 | Designates aligner. | Alignment is not performed automatically after carry-in. Send "MOV:ALIGN;", if necessary. |
| LLA | Designates Upper LLA. | Slot 1 is targeted. |
| LLAxx | Designates Upper LLA and slot. | xx is slot number. (01 to) |
| LLB | Designates Upper LLB. | Slot 1 is targeted. |
| LLBxx | Designates Upper LLB and slot. | xx is slot number. (01 to) |

- Parameter 2 of UNLOAD

Designates the finger which is used for carrying in.

| Parameter 2 | Descriptions | Remarks |
| --- | --- | --- |
| ARM1 | Designates upper finger. | Upper finger carries in a wafer. |
| ARM2 | Designates lower finger. | Lower finger carries in a wafer. |

Note: The single arm robot does not have lower finger. Therefore, if "ARM2" is designated, error message "NAK:MSG" is returned.

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

TRANS

-Function: Designates the transfer source and transfer destination to transfer a wafer.

- Request for TRANS (E/C => EFEM PC)

| MOV | : | TRANS | / | Parameter 1 | > | Parameter 2 | / | Parameter 3 | > | Parameter 4 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of TRANS (E/C <= EFEM PC)

| ACK | : | TRANS | / | Parameter 1 | > | Parameter 2 | / | Parameter 3 | > | Parameter 4 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of TRANS (E/C <= EFEM PC)

| INF | : | TRANS | / | Parameter 1 | > | Parameter 2 | / | Parameter 3 | > | Parameter 4 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of TRANS (E/C => EFEM PC)

| ACK | : | TRANS | / | Parameter 1 | > | Parameter 2 | / | Parameter 3 | > | Parameter 4 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alignment completion information of TRANS (E/C <= EFEM PC) Note: This is reported only when Parameter 4 is "ALIGN".

| INF | : | ALIGN | ; |
| --- | --- | --- | --- |

- Alignment completion response of TRANS (E/C => EFEM PC)

| ACK | : | ALIGN | ; |
| --- | --- | --- | --- |

- Parameter 1 and Parameter 4 of TRANS

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| Pnxx | Designates load port and slot. | n is port number. (1 to), xx is slot number. (01 to) |
| ALIGN | Designates aligner. | Alignment is not performed automatically before carry-out. Send "MOV:ALIGN;", if necessary. |
| ALIGN2 | Designates aligner. | Alignment is performed automatically before carry-out. |
| LLA | Designates Upper LLA. | Slot 1 is targeted. |
| LLAxx | Designates Upper LLA and slot. | xx is slot number. (01 to) |
| LLB | Designates Upper LLB. | Slot 1 is targeted. |
| LLBxx | Designates Upper LLB and slot. | xx is slot number. (01 to) |

- Parameter 2 and Parameter 3 of TRANS

Parameter 2 designates the finger used for carrying out. Parameter 3 designates the finger used for carrying in.

| Parameter 2, 3 | Descriptions | Remarks |
| --- | --- | --- |
| ARM1 | Designates upper finger. | Upper finger is used. |
| ARM2 | Designates lower finger. | Lower finger is used. |

Note: The single arm robot does not have lower finger. Therefore, if "ARM2" is designated, error message "NAK:MSG" is returned.

- Parameter 4 of TRANS Designates transfer destination.

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| Pnxx | Designates load port and slot. | n is port number. (1 to), xx is slot number. (01 to) |
| ALIGN | Designates aligner. | Alignment is performed automatically after carry-in. |
| ALIGN2 | Designates aligner. | Alignment is not performed automatically after carry-in. Send "MOV:ALIGN;", if necessary. |
| LLA | Designates Upper LLA. | Slot 1 is targeted. |
| LLAxx | Designates Upper LLA and slot. | xx is slot number. (01 to) |
| LLB | Designates Upper LLB. | Slot 1 is targeted. |
| LLBxx | Designates Upper LLB and slot. | xx is slot number. (01 to) |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

CHANGE

-Function: A wafer is exchanged at designated position.

- Request for CHANGE (E/C => EFEM PC)

| MOV | : | CHANGE | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of CHANGE (E/C <= EFEM PC)

| ACK | : | CHANGE | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Completion information of CHANGE (E/C <= EFEM PC)

| INF | : | CHANGE | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of CHANGE (E/C => EFEM PC)

| ACK | : | CHANGE | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alignment completion information of CHANGE (E/C <= EFEM PC) Note: This is reported only when Parameter 1 is "ALIGN".

| INF | : | ALIGN | ; |
| --- | --- | --- | --- |

- Alignment completion response of CHANGE (E/C => EFEM PC)

| ACK | : | ALIGN | ; |
| --- | --- | --- | --- |

- Parameter 1 of CHANGE

Designates a destination to exchange a wafer.

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| ALIGN | Designates aligner. | Alignment is performed automatically after carry-in. |
| ALIGN2 | Designates aligner. | Alignment is not performed automatically after carry-in. Send "MOV:ALIGN;", if necessary. |
| LLA | Designates Upper LLA. | Slot 1 is targeted. |
| LLAxx | Designates Upper LLA and slot. | xx is slot number. (01 to) |
| LLB | Designates Upper LLB. | Slot 1 is targeted. |
| LLBxx | Designates Upper LLB and slot. | xx is slot number. (01 to) |

- Parameter 2 and Parameter 3 of CHANGE

Parameter 2 designates the finger used for carrying out. Parameter 3 designates the finger used for carrying in.

| Parameter 2, 3 | Contents | Remarks |
| --- | --- | --- |
| ARM1 | Designates upper arm. | Upper arm is used. |
| ARM2 | Designates lower arm. | Lower arm is used. |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

ALIGN

-Function: A wafer on the aligner is aligned. In case of "SET" message, notch stop position after alignment is designated.

- Request for ALIGN (E/C => EFEM PC)

| MOV | : | ALIGN | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of ALIGN (E/C <= EFEM PC)

| ACK | : | ALIGN | ; |
| --- | --- | --- | --- |

- Completion information of ALIGN (E/C <= EFEM PC)

| INF | : | ALIGN | ; |
| --- | --- | --- | --- |

- Completion response of ALIGN (E/C => EFEM PC)

| ACK | : | ALIGN | ; |
| --- | --- | --- | --- |

- Parameter of ALIGN

Designates the notch stop position when alignment is completed. In case of "SET" message, parameter cannot be omitted.

| Parameter 1 | Descriptions | Remarks |
| --- | --- | --- |
| Pn | Designates load port and slot. | n is port number. (1 to) |
| LLA | Position to be stored at Upper LLA. |  |
| LLB | Position to be stored at Upper LLB. |  |
| A***.** | Designates angle. | ***.** is angle. (-359.00 to 359.99) |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

HOME

-Function: Moves each unit to the home position (waiting position before transfer).

- Request for HOME (E/C => EFEM PC)

| MOV | : | HOME | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of HOME (E/C <= EFEM PC)

| ACK | : | HOME | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion information of HOME (E/C <= EFEM PC)

| INF | : | HOME | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Completion response of HOME (E/C => EFEM PC)

| ACK | : | HOME | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Parameter of HOME Parameter cannot be omitted.

| Parameter | Descriptions |  |  | Remarks |
| --- | --- | --- | --- | --- |
| ROB | Designates | robot. |  |  |
| Pn | Designates | load port | n. | n is port number. (1 to) |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

HOLD

-Function: Stops motion axes of EFEM temporarily. (Stop upon deceleration) This is invalid for the load port operation.

There is no parameter.

- Request for HOLD (E/C => EFEM PC)

| MOV | : | HOLD | ; |
| --- | --- | --- | --- |

- Normal response of HOLD (E/C <= EFEM PC)

| ACK | : | HOLD | ; |
| --- | --- | --- | --- |

- Completion information of HOLD (E/C <= EFEM PC)

| INF | : | HOLD | ; |
| --- | --- | --- | --- |

- Completion response of HOLD (E/C => EFEM PC)

| ACK | : | HOLD | ; |
| --- | --- | --- | --- |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

RESTR

-Function: Resumes motion stopped temporarily by "HOLD" message.

There is no parameter.

- Request for RESTR (E/C => EFEM PC)

| MOV | : | RESTR | ; |
| --- | --- | --- | --- |

- Normal response of RESTR (E/C <= EFEM PC)

| ACK | : | RESTR | ; |
| --- | --- | --- | --- |

| Motion | Status acquisition |
| --- | --- |
| ○ |  |

ABORT

-Function: Aborts and ends the motion stopped temporarily by "HOLD" message.

If the finger is being extended, the motion is stopped after returning the finger to origin. There is no parameter.

- Request for ABORT (E/C => EFEM PC)

| MOV | : | ABORT | ; |
| --- | --- | --- | --- |

- Normal response of ABORT (E/C <= EFEM PC)

| ACK | : | ABORT | ; |
| --- | --- | --- | --- |

- Completion information of ABORT (E/C <= EFEM PC)

| INF | : | ABORT | ; |
| --- | --- | --- | --- |

- Completion response of ABORT (E/C => EFEM PC)

| ACK | : | ABORT | ; |
| --- | --- | --- | --- |

| Motion | Status acquisition |
| --- | --- |
|  | ○ |

ERROR

-Function: Reports that an error occurred on EFEM.

- Report of ERROR (E/C <= EFEM PC) Reports to E/C that an error occurred.

| INF | : | ERROR | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Response of ERROR report (E/C => EFEM PC)

| ACK | : | ERROR | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- ERROR clear (E/C => EFEM PC)

Sends from E/C to reset the error. Only recoverable error is reset.

| SET | : | ERROR | / | CLEAR | ; |
| --- | --- | --- | --- | --- | --- |

- Response of ERROR clear (E/C <= EFEM PC)

| ACK | : | ERROR | / | CLEAR | ; |
| --- | --- | --- | --- | --- | --- |

- Request for ERROR acquisition(E/C=>EFEM)

| GET | : | ERROR | ; |
| --- | --- | --- | --- |

- Parameter 1 of ERROR

| Parameter 1 | Descriptions |
| --- | --- |
| E84_TIMEOUT1 | E84 TR_REQ timeout (TP1) |
| E84_TIMEOUT2 | E84 BUSY ON timeout (TP2) |
| E84_TIMEOUT3 | E84 carrier carrying-in timeout (TP3) |
| E84_TIMEOUT4 | E84 carrier carrying-out timeout (TP3) |
| E84_TIMEOUT5 | E84 BUSY OFF timeout (TP4) |
| E84_TIMEOUT7 | E84 VALID OFF timeout (TP5) |
| E84_TIMEOUT8 | E84 CONTINUE timeout |

- Parameter 2 of ERROR

| Parameter 2 | Descriptions |
| --- | --- |
| ROB |  |
| Pn |  |
| Aligner |  |
| UNDEFINITION |  |

| Motion | Status acquisition |
| --- | --- |
| ○ | ○ |

CLAMP

-Function: Acquires on output, off output, or clamp state during clamping operation of the robot and aligner.

Note: During normal sequence, do not use this message because clamp on/off and status check is automatically performed.

- Request for CLAMP (E/C => EFEM PC)

| SET | : | CLAMP | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of CLAMP (E/C <= EFEM PC)

| ACK | : | CLAMP | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Request for CLAMP status acquisition (E/C => EFEM PC)

| GET | : | CLAMP | / | Parameter 1 | ; |
| --- | --- | --- | --- | --- | --- |

- Response of CLAMP status acquisition (E/C <= EFEM PC)

| ACK | : | CLAMP | / | Parameter 1 | ; |
| --- | --- | --- | --- | --- | --- |

- Report of CLAMP status (E/C <= EFEM PC)

| INF | : | CLAMP | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Response of CLAMP status report (E/C => EFEM PC)

| ACK | : | CLAMP | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of CLAMP

| Parameter 1 | Descriptions |
| --- | --- |
| ARMa | a is finger number. (Upper finger=1, Lower finger=2). |
| ALIGN | Designates aligner. |

- Parameter 2 of CLAMP

| Parameter 2 | Descriptions |
| --- | --- |
| ON | (1) Instructs clamp when requested. (5) Turns on clamp sensor when reporting state. |
| OFF | (1) Instructs unclamp when requested. (5) Turns off clamp sensor when reporting state. |

| Motion | Status acquisition |
| --- | --- |
|  | ○ |

STATE

-Function: Acquires status of system state, etc.

- Request for STATE information acquisition (E/C => EFEM PC)

| GET | : | STATE | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Response of STATE information acquisition (E/C <= EFEM PC)

| ACK | : | STATE | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Report of STATE information (E/C <= EFEM PC)

| INF | : | STATE | / | Parameter | / | Data | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Response of STATE information report (E/C => EFEM PC)

| ACK | : | STATE | / | Parameter | / | Data | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Report of STATE event  （E/C <= EFEM PC）

In case update event is valid, report is sent when the status of the connected devices changed.

| EVT | : | STATE | / | Parameter | / | Data | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter of STATE

| Parameter | Contents |
| --- | --- |
| VER | Acquisition of version information for system program |
| TRACK | Wafer presence check on finger and aligner |

- Data of STATE

|  |  | . |  |  |  | ( |  |  |  |  | - |  |  | - |  |  | ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Format of data changes according to the parameter. (7-1) Parameter 2 format of STATE/VER

Version	Date

(Ex.) Ver1.0.0.1: Report of January 1, 2010.

INF:STATE/VER/1.0.0.1(2010-01-01);

(7-2) Parameter 2 format of STATE/TRACK All response contents are ASCII code.

| (1) | / | (2) | / | (3) |
| --- | --- | --- | --- | --- |

"(1)" shows the wafer presence on upper finger, "(2)" on lower finger, and "(3)" on aligner.

| (1) to (3) | Descriptions |
| --- | --- |
| ???? | Presence uncertain |
| NONE | No wafer |
| Pnxx | Wafer is present in slot xx at transfer source Pn. |
| ULLnxx | Wafer is present in slot xx at transfer source UpperLLn. |
| LLLnxx | Wafer is present in slot xx at transfer source LowerLLn. |
| DPnxx | Wafer is present in slot xx at transfer source DPn. |
| EXST | Wafer is present at unknown transfer source. |

Note: In case of single arm robot, "(2)" is always "????".

| Motion | Status acquisition |
| --- | --- |
|  | ○ |

MODE

-Function: Changes E84 automatic transfer setting, or acquires current control status.

- Request for MODE setting (E/C => EFEM PC)

| SET | : | MODE | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of MODE (E/C <= EFEM PC)

| ACK | : | MODE | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Request for MODE status acquisition (E/C => EFEM PC)

| GET | : | MODE | / | Parameter 1 | ; |
| --- | --- | --- | --- | --- | --- |

- Response of MODE status acquisition (E/C <= EFEM PC)

| ACK | : | MODE | / | Parameter 1 | ; |
| --- | --- | --- | --- | --- | --- |

- Report of MODE status (E/C <= EFEM PC)

| INF | : | MODE | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Response of MODE status report (E/C => EFEM PC)

| ACK | : | MODE | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of MODE Designates a target port.

| Parameter 1 | Descriptions |
| --- | --- |
| ALL | All ports are set as a batch. (Cannot be designated for GET) |
| Pn | n is port number. (1 to) |

- Parameter 2 of MODE

Designates control method for setting request. Current control status means for status acquisition.

| Parameter 2 | Descriptions |
| --- | --- |
| AUTO | Performs E84 sequence automatically. |
| MANUAL | Controls E84 sequence only by TRANSREQ message. |

| Motion | Status acquisition |
| --- | --- |

SIGOUT

-Function: Changes output status of signals such as signal tower and load port indicator.

- Request for SIGOUT setting (E/C => EFEM PC)

| SET | : | SIGOUT | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Normal response of SIGOUT (E/C <= EFEM PC)

| ACK | : | SIGOUT | / | Parameter 1 | / | Parameter 2 | / | Parameter 3 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of SIGOUT

Designates the target for signal output.

| Parameter 1 | Descriptions |
| --- | --- |
| IO | System Output |

- Parameter 2 of SIGOUT

| Parameter 2 | Descriptions |
| --- | --- |
| n | System Output Index |

- Parameter 3 of SIGOUT Designates signal status to output.

| Parameter 3 | Descriptions |
| --- | --- |
| ON | Turn on. |
| OFF | Turn off |

| Motion | Status acquisition |
| --- | --- |
|  | ○ |

SIGSTAT

-Function: Acquires input and output status of I/O signal.

- Request for SIGSTAT status acquisition (E/C => EFEM PC)

| GET | : | SIGSTAT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of SIGSTAT status acquisition (E/C <= EFEM PC)

| ACK | : | SIGSTAT | / | Parameter | ; |
| --- | --- | --- | --- | --- | --- |

- Report of SIGSTAT status (E/C <= EFEM PC)

| INF | : | SIGSTAT | / | Parameter | / | Data 1 | / | Data 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Response of SIGSTAT status report (E/C => EFEM PC)

| ACK | : | SIGSTAT | / | Parameter | / | Data 1 | / | Data 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Event report of SIGSTAT (E/C <= EFEM PC)

In case I/O update event is effective, this is sent when I/O status is changed.

| EVT | : | SIGSTAT | / | Parameter | / | Data 1 | / | Data 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter of SIGSTAT

Designates the target for acquisition.

| Parameter | Descriptions |
| --- | --- |
| SYSTEMn | System I/O n is index for I/O ((0 to) |
| Pn | n is port number. (1 to |

- Data 1

(7-1) Format of Data 1 for SIGSTAT/SYSTEMn

| DATA1 | Descriptions |
| --- | --- |
| ON | input signal is valid |
| OFF | input signal is invalid |

(7-2) Format of Data 1 for SIGSTAT/Pn

| DATA1 | Descriptions |
| --- | --- |
| PRESENT | Box present |
| EMPTY | Box removed |

- Data 2 of SIGSTAT/SYSTEMn

| DATA2 | Descriptions |
| --- | --- |
| ON | output signal is valid |
| OFF | output signal is invalid |

| Motion | Status acquisition |
| --- | --- |
|  | ○ |

CSTID

-Function: Reads the carrier ID for a carrier on a designated port.

- Request for CSTID acquisition (E/C => EFEM PC)

| GET | : | CSTID | / | Parameter 1 | ; |
| --- | --- | --- | --- | --- | --- |

- Normal response of CSTID (E/C <= EFEM PC)

| ACK | : | CSTID | / | Parameter 1 | ; |
| --- | --- | --- | --- | --- | --- |

- Request for CSTID setting (E/C => EFEM PC)

| SET | : | CSTID | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Response of CSTID setting (E/C <= EFEM PC)

| ACK | : | CSTID | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of CSTID (E/C <= EFEM PC)

| INF | : | CSTID | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Completion response of CSTID (E/C => EFEM PC)

| ACK | : | CSTID | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Event report of CSTID (E/C <= EFEM PC)

The event is sent when the carrier ID reading is performed while opening the carrier.

| EVT | : | CSTID | / | Parameter 1 | / | Parameter 2 | ; |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Parameter 1 of CSTID Designates the target port.

| Parameter 1 | Descriptions |
| --- | --- |
| Pn | n is port number. (1 to) |

- Parameter 2 of CSTID

Shows the reading result. In the case of setting, designates the ID to be set.

| Parameter 2 | Descriptions |
| --- | --- |
| ***…*** | Carrier ID that was read |
| NOREAD | No carrier ID reading performed, Reading failed |

- Parallel I/O Table

  - Input

  - Output

  - Parallel I/O timing chart

- SETUP Sequence

(ACK is omitted except READY/COMM.)

E/C	EFEM	Operator

(2) EFEM start-up

INF:READY/COMM;	(3) Send INF every xx sec

until response of INF is returned.

ACK:READY/COMM;	(4) Communication established

MOV:INIT;

(5) Start EFEM initialization

INF:INIT;

(6) Initialization completed

MOV:ORGSH;

(10) Start EFEM origin search

INF:ORGSH;

(11) EFEM origin search completed

(1) POWER ON

- Request for origin search

- Check system state

- Origin search