| DOCUMENT TITLE: |  |  |
| --- | --- | --- |
| EFEM Communication Interface |  |  |
|  | DOCUMENT NUMBER: | REV: |
|  |  | 0.1 |
|  | SYSTEM(S) or MODULE(S): |  |
|  | JUST TEMPLATE DOC FOR WTS |  |

| REV | DATE | ORIGINATOR | CHANGE DESCRIPTION |
| --- | --- | --- | --- |
| 0.1 | 2023/3/13 | Longhua | First Release |

Fortrend Engineering Corporation reserves all copyright related to the document.

Note: This is a protocol demo document, and the command described in this document may be added or removed in the actual distribution version.

Table of Contents

1	Scope	4

2	Abbreviate Word	4

3	System Specification	4

4	Control Mode	4

5	Protocol to Communicate with Host	5

6	Message Definition	8

6.1	Host to EFEM	8

6.2	EFEM to Host	10

7	Control Sequence Example	11

7.1	System Reset	11

7.2	Sub-System Command	13

7.3	System Command	14

7.4	Use Case	15

8	Sub-System Command/Response	15

8.1	WHR	16

8.2	FTR	32

8.3	Opener	34

8.4	Pusher	37

8.5	HV	39

8.6	Aligner	41

8.7	Loadport	42

8.8	RFID	43

8.9	FOUP Buffer Clamp	44

9	System Command	46

10	System Response	47

11	Defined Command	47

12	Event	48

13	Station Defined	49

14	Error Code Definition (4 Bytes)	50

14.1	System Error (Type=0)	50

15	EFEM Operation Guide	52

15.1	System Start	52

15.2	Error Recovery	52

15.3	Get/Put Carrier	52

- Scope

This document is used to define the communication interface to allow remote controller (Host) to control the EFEM Unit.

- Abbreviate Word

GUI – Graphic User Interface in the EFEM to control the Transport Unit.

EFEM – Equipment Front End Module.

- System Specification

<按实际布局>

- Control Mode

The EFEM can be controlled through GUI or Host. It has ‘Control Mode’ state inside. When it’s at ‘Local Mode’, it can just be controlled by GUI. If it’s at ‘Remote Mode’, it can just be controlled by Host. But Command Type is ‘System Command’ (except Pause Operation and Resume Operation) will be accepted no matter it’s at Local or Remote Mode, unless it is rejected by other reasons.

The default Control Mode is ‘Remote Mode’.

If EFEM is at Local Mode, it won’t transfer any Event to Host except ‘Control Mode Changed’.

- Protocol to Communicate with Host

Control of  EFEM  is established through a series of messages. Each command is identified through a Command Type byte and 2 Command ID bytes and possibly followed by parameters.  If the command can be processed by the  EFEM , an ACK is returned: if not,  EFEM  generates a NAK.  If the command is acknowledged, several informational messages may be generated by  EFEM  including completion and error status.

The message transfer protocol is HEX TCP/IP.

HEX:

Message is sent/ received via serial communication port. The serial line characteristics of the interface are: Baud=9600, Parity No, Data Bits=8, Stop Bit 1.

Message format is based upon the Block Transfer Protocol specified in SEMI E4-91. A general description of this format follows:

- A message cycle consists of a unit (the requesting unit) requesting to send, the other (receiving unit) replying that it is ready to receive, a transmission of the message by the requesting unit and finally an acknowledgment of reception without errors and also the ability to perform the command.   EFEM  acts as the master and the host as the slave.

- Following the request to send, a length byte is transmitted from the requesting unit.  The length is equal to the number of characters in the message block, and may take any value from 1 to 254.

- Following the length byte, a message block is transmitted.

- After the message block is sent, 2 bytes of checksum are transmitted (most significant byte first).  This represents the sum of all the bytes contained in the message block.

- Special characters used to support this format are:

ENQ  (0x05) Request to send. 	ACK  	(0x06) Message received OK is being executed.

EOT  (0x04) Ready to receive.	NACK (0x15) Failure in message reception.

- In the case of errors,  EFEM  will retransmit the message a configurable number of times.  After a configurable number of tries,  EFEM  will give up on the message and wait for communication to be established from the host.

- While waiting for either an EOT or an ACK/NACK,  EFEM  will ignore any other characters received.  Thus, in the case of contention, the Host should accept  EFEM  messages.

- Two time-outs are active during the transmission of a message.  The first is used for ENQ-EOT, and is referred to as the T2 time-out.  The second is the inter character time-out, T1 time-out.  If either of these is violated, a NAK and an error will be generated.

HEX TCPIP:

Message is sent/ received via Ethernet port. Message format is the same as HEX. A general description of this format follows:

- Not to send the unit (the requesting unit) requesting to send and the other (receiving unit) replying that it is ready to receive as HEX.

- A length byte is transmitted from the requesting unit.  The length is equal to the number of characters in the message block, and may take any value from 1 to 255.  EFEM  acts as the server and the host as the client.

- Following the length byte, a message block is transmitted.

- Not to send checksum after the message block is sent

Host initiates a message cycle (HEX)

Messages – The messages is a character string consisting of the following information.

(1) The length bye is an unsigned character indicating the number of bytes in the message block.

(2) A message block includes interested information that is defined below.

(3) After the message block is two checksum bytes (most significant byte first).

EFEM initiates a message cycle (HEX)

Messages follow the same definitions as in the previous example.

- Message Definition

  - Host to EFEM

Message Structure

Byte 1~4: Command Index (Byte 1: MByte; Byte 2: 2nd Byte; Byte 3: 3rd Byte; Byte 4: LByte)

Any response that  EFEM  sends to Host relatives to this command will use the Command Index as Response Index to inform Host the source of the response. It should start from 0x00000001 and should be different from each other. 0x00000000 will be used by  EFEM  to indicate this message is an event and has no relative command like Carrier Present event.

Byte 5: Command Type

0x00:	Sub-System Command

Command to send to Sub-System directly.

Byte 6:		Sub-System (see Station Definition)

Byte 7~:	Command ID & Parameters (see Sub-System Command/Response)

Response:	Sub-System Command Response

0x01:	Defined Command

Command that is defined by EFEM to let the Host be able to issue a serial commands.

Byte 6~:	Defined Command ID (Byte 6: MByte; Byte 7: LByte) & Parameters (see Defined Command)

Response:	Defined Command Response

0x02:	System Command

Command to get the status of  EFEM  or change state. It won’t generate Acknowledge Response unless it is rejected for some reason.

Byte 6~:	System Command ID (Byte 6: MByte; Byte 7: LByte) & Parameters (see System Command)

Response:	System Response

For any command except System Command,  EFEM  will return Acknowledge Response immediately when it receives a command to indicate it accepts the command or rejects it. If  EFEM  accepts the command, it starts running the command. When it finishes, it will return the cooperative response.

System Command will get System Response unless its parameter(s) or condition is wrong. If it has wrong parameter(s), it will get Acknowledge Response with Not Acknowledge result.

  - EFEM to Host

Message Structure

Byte 1~4: Response Index (Byte 1: MByte; Byte 2: 2nd Byte; Byte 3: 3rd Byte; Byte 4: LByte)

It will be the same as the Command Index of the command relative if it has. Otherwise it will be 0.

Byte 5: Response Type

0x00:	Acknowledge Response

Byte 6:		Result

0x00:		Acknowledge

0x01:		Not Acknowledge

Byte 7 ~ 10:	Error Code (see Error Code Definition (4 Bytes))

0x01:	Sub-System Command response

Byte 6:		The Sub-System (see Station Definition)

Byte 7~:	The Response ID & Parameters (see Sub-System Command/Response)

Or

Byte 6~9:	System Error code (Byte 6: 0x80) (see Error Code Definition (4 Bytes))

0x02:	Defined Command response

Byte 6:		Result

0x00:		Command completed.

0x01:		Error Detected.

Byte 7 ~ 10:	Error Code (see Error Code Definition (4 Bytes))

0x03:	System Command Response

Byte 6~:	System Response ID (Byte 6: MByte; Byte 7: LByte) & Parameters (see System Response)

0x04:	Event

Byte 6~:	Event ID (Byte 6: MByte; Byte 7: LByte) & Parameters (see Event)

- Control Sequence Example

  - System Reset

Case 1: Command is acknowledged by EFEM and runs successfully.

Case 2: Command is rejected because current Control Mode is ‘Local Mode’.

Case 3: Command is acknowledged and starts running but fails when Robot ‘Software Reset’.

  - Sub-System Command

Case 1: Get Object command is acknowledged and runs successfully.

Case 2: Get Object command is rejected because robot has error and needs to be reset.

Case 3: Get Object command is acknowledged and starts running, but fails.

  - System Command

Case 1: Set Control Mode to ‘Remote Mode’ and succeed

Case 2: Set Control Mode to ‘Remote Mode’ but is rejected because System is busy

  - Use Case

Host Initial Process

Here supply a Use Case to show how Host can control EFEM and get the critical information when it just connects to EFEM.

Subject: Initial EFEM while connected.

Primary actor: Host

Second actor: None

Level: See level

Pre-condition: None

Main success scenario

- Host sends a command to change Control Mode to ‘Remote’.

- Host sends a command to set current time.

- Host sends a command to read Robot end-effecter carrier present status.

- Host sends a command to reset the Transport Unit.

Extension

1.a. EFEM sends a NACK response.

1. Host waits until EFEM completes current command then changes Control Mode again.

- Sub-System Command/Response

For every Sub-System, except the Command/Response/Error Code defined in each sub-system.

WHR

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Robot to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Robot to return inner status. |
| Get Wafer | 0x03 | P1:Station P2: slot(not use) P3:Arm 0:arm1 1:arm2 | 0x62:Completed 0x63: Error Detected. | This command instructs Robot to get a wafer from specified position |
| Put Wafer | 0x04 | same as up | 0x62:Completed 0x63: Error Detected. | This command instructs Robot to put a wafer to specified position. |
| Ready Get Wafer | 0x05 | same as up | 0x62:Completed 0x63: Error Detected. | This command instructs Robot ready to get a wafer from specified position |
| Ready Put Wafer | 0x06 | same as up | 0x62:Completed 0x63: Error Detected. | This command instructs Robot ready to put a wafer to specified position. |
| Mapping | 0x07 | none | 0x62:Completed 0x63: Error Detected. | Mapping |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:: arm has wafer or not bit 0: arm1 bit 1: arm2 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

WHR Error Code Definition

Error Type: 1

1:	Appointed step number does not exist*

2:	Incomplete Origin process*

3:	The condition was not achieved during IF command even after appointed time*

4:	Input emergency stop value*

5:	Program area setting error*

6:	System burg*

7:	PGM is over 2000 steps*

8:	Over-Run state*

9:	Barrier check*

10:	Extended board in Motor axis is not connected*

11:	Driver is not Ready*

12:	Driver is alarmed(Code is 99)*

13:	Parameter or flag number is not among 1~96*

14:	Over the RAM area*

15:	Turbo parameter communication error*

16:	The contents of Parameter area is not right*

17:	Communication error with HOST*

18:	GOTO Step Moving setting is wrong*

19:	The variable value as Coordinates value overflows 3 bytes*

20:	CALL Nest overflows 10*

21:	CALL return error (System bug)*

22:	Call the program recursively*

23:	Appointed program number is not existed*

24:	Other commands are existed except for PTP and MATRIX*

25:	Appointed program number is not existed*

26:	Program (PGM=999) is not existed*

27:	The channel no. for OUT command:	 etc is wrong*

28:	Execution Command for the appointed program is not existed*

29:	Zero division occurs due to the variable calculation*

30:	Unapproved command. (optional command.)*

31:	Floating point calculation error*

32:	Not able to start the program*

33:	Command not registered Atom*

34:	Not able to check the appointed table number*

35:	The program to execute is sub-program*

36:	The robot stopped during processing origin*

37:	Variable DI:	 DO value is wrong in BCD*

38:	Check sum of program area is wrong*

39:	Check sum or parameter area is wrong*

40:	Atom Encoder value is not correct*

41:	Atom communication error*

42:	The program to be executed is main program*

43:	Unaccepted command(XX) to Atom*

44:	Atom is not responding to new command*

45:	System error*

46:	There's a command that emergency stop is unapplied*

47:	Motor number Error*

48:	Low Memory Back up battery (Warning Mark)*

49:	EEPROM WRITE ERROR*

50:	Hardware is not matched with FirmWare*

51:	No Fics-BIOS*

52:	Internal error*

53:	FICS.PGM file read error*

54:	FICS.PAR file read error*

100:	Table number of Matrix exceeds 5*

122:	The speed parameter of DDA is too fast*

124:	DDA speed number is not existed*

125:	No.1 and 2 are equal point*

126:	Lack of Memory while executing DDA*

127:	No.1 and 3 are equal point*

128:	No.2 and 3 are equal point*

134:	Wrong Layer data*

135:	Zone is too large or pitch is too small*

136:	Zone is too small or pitch is too large*

151:	Incorrect communication format*

152:	Appointing DM is not right*

157:	The factor of macro is not right*

161:	The program written in table ref. command is different*

162:	Difference between table ref. command and ref. program type*

163:	The step does not exist*

170:	No track program*

171:	There's a command not to be written to Track program*

172:	There are other commands except for LINE/ARC*

173:	Track executing axis is not organized*

174:	Serial TRACK command is over 32*

175:	Track whole step number is over 64*

201:	Data range error*

202:	There's a wafer before GET Motion*

203:	No wafer before PUT Motion*

204:	No wafer after GET Motion*

205:	There's a wafer after PUT Motion*

206:	Sensor Vacuum related Error during GET Motion*

207:	Sensor Vacuum related error during PUT motion*

208:	Arm is not positioned at HOME*

209:	Not supported function*

210:	Sense the Wafer projection*

211:	Prohibit the Arm moving*

212:	There's wafer at origin*

213:	2-point teaching has big difference*

214:	Error occurs during 1 Hand change*

215:	Error occurs during 5 Hand change*

216:	Mapping arm HOME position error*

217:	Rotating axis HOME position error*

218:	Hand sensor sensing error*

220:	Unknown parameter*

223:	Sensor fails to perceive in limited time(3Sec)*

224:	No wafer of Vacuum not detect after GET*

225:	Wafer of present detect after PUT*

226:	No wafer of Vacuum not detect during GET*

227:	Wafer of Vacuum detect during PUT*

232:	Wafer of Present detect before GET*

233:	No Wafer of Present not detect before PUT*

234:	No Wafer of Present no detect after GET*

235:	Wafer of Present detect after PUT*

236:	No Wafer of Present not detect during GET*

237:	Wafer of Present detect during PUT*

241:	Blade position is not correct*

242:	Wafer of Present and Vacuum detect before GET* GET*

243:	No Wafer of Present and Vacuum not detect before PUT*

244:	No Wafer of Present and Vacuum no detect after GET*

246:	No Wafer of Present and Vacuum not detect during GET*

247:	Wafer of Present and Vacuum detect during PUT*

241:	Blade position is not correct*

251:	Mapping is not performed, needed to check Pusher home sensor*

252:	No mapping data*

253:	Latch Mutex error*

300:	Pusher on gripper is not released. Check Pusher home sensor*

301:	Invalid command*

311:	Invalid data*

312:	Invalid station*

313:	Invalid hand*

314:	Invalid slot*

315:	Invalid teaching index*

316:	Invalid point data*

317:	Invalid hand type*

318:	Box is not coming out*

319:	Parameter is not set on the station*

321:	Invalid coordinate type*

331:	Invalid argument*

333:	Invalid format*

334:	Invalid location format*

335:	Invalid profile*

341:	Wrong point data command*

342:	Wrong AWC data*

343:	No AWC Station*

344:	Wrong latch data*

345:	No latch data*

351:	No data*

352:	Not home location*

353:	Cannot retract arm*

354:	Undetected Vacuum *

355:	No box*

356:	Cannot ungrip*

357:	Detected double box*

359:	Cannot origin. Needed manual operation*

360:	No supply air*

361:	Undetected arm home sensor*

371:	Got new command while Robot is on move*

372:	Lag is detected from Z axis position sensor*

378:	Cannot flip hand*

380:	Stopped cooling-fan in comntroller*

381:	Stoped cooling-fan in Robot*

400:	No supply hand grip air*

401:	Full receiving buffer*

402:	Full sending buffer*

403:	Connection error with RIO Board*

500:	No memory available*

501:	System internal consistency error*

502:	Invalid argument*

503:	FIFO overflowed*

504:	Not implemented*

505:	Missing argument*

506:	Invalid auto execute mode*

507:	Too many*

508:	Protection error*

509:	Read only*

510:	Operating system error code*

511:	Option not enabled*

512:	License not installed*

513:	Invalid password*

514:	Cancelled*

515:	No system clock interrupys*

516:	System clock interrupts too slow*

517:	System clock interrupts too fast*

518:	Invalid task configuration*

519:	Incompatible FPGA version*

520:	Not configured*

521:	Invalid robot type*

530:	Invalid device*

531:	Undefined device*

532:	Invalid device unit*

533:	Undefined device unit*

534:	Device already in use*

538:	Too many devices*

539:	No physical device mapped*

540:	Timeout waiting for device*

541:	Date/time not set*

542:	Invalid date/time specification*

544:	Device not found*

545:	NVRAM not responding*

546:	NVRAM invalid response*

547:	NVRAM data invalid*

548:	Real-time-clock disabled*

549:	Real-time-clock battery failed*

550:	Device not ready*

551:	Invalid device command*

552:	i2c device failure*

553:	Device full*

554:	MCP not recognized*

555:	SIO device failure*

556:	Device not enabled*

560:	Device I/O error*

561:	No I/O pending

562:	No message buffers available*

563:	Undefined node*

564:	Less data than expected*

565:	Unexpected end of file*

566:	Input too long*

567:	Output too long*

568:	Output too long*

569:	Can't open file for write*

570:	File already exists*

571:	Can't create directory*

572:	Error writing file*

573:	Error reading file*

574:	Buffer already in use*

575:	Invalid input character*

576:	Invalid file name*

577:	File path too long*

578:	Invalid data checksum*

579:	Invalid file format*

580:	File not open*

581:	Invalid file type*

582:	File interlocked*

583:	No data available*

584:	No timestamp available*

585:	Latch input overrun*

586:	Latch data overrun*

590:	Thread execution aborted*

592:	Undefined thread*

594:	Missing quote mark*

595:	Value too small*

596:	Value too large*

597:	Value out-of-range*

598:	Value infinite or NAN*

599:	Division by 0*

600:	Arithmetic overflow*

601:	Singular matrix*

602:	Invalid syntax*

603:	Symbol too long*

604:	Unknown command*

605:	Invalid procedure step*

606:	Ambiguous abbreviation*

607:	Invalid number format*

608:	Missing parentheses*

609:	Illegal use of keyword*

610:	Unexpected character in expression*

611:	Conflict with reserved keyword*

612:	Unexpected text at end of line*

613:	Invalid statement label*

614:	Invalid End keyword*

615:	Unknown data type*

616:	Data type required*

617:	Cannot redefine symbol*

618:	Procedure too long*

619:	Undefined symbol*

620:	Invalid symbol type*

621:	Unmatched parentheses*

622:	Invalid procedure end*

623:	Not a top-level statement*

624:	Object not bound to class*

625:	Too many nested blocks*

626:	Duplicate statement label*

627:	Too many errors:	 compile cancelled*

628:	Invalid data type*

629:	Cannot change built-in symbol*

630:	Empty procedure*

631:	Argument mismatch*

632:	Compilation errors*

633:	Invalid project file*

634:	Invalid start procedure*

635:	Project already exists*

636:	Interlocked for read*

637:	Interlocked for write*

638:	No matching control structure*

639:	Thread already exists*

640:	Invalid when thread active*

641:	Timeout starting thread*

642:	Timeout stopping thread*

643:	Project not compiled*

644:	Thread execution complete*

645:	Thread stack too small*

646:	Member not shared*

647:	Object not instantiated*

648:	No Get Property defined*

649:	Undefined value*

650:	Invalid assignment*

651:	Cannot have list of variables*

652:	Location not a Cartesian type*

653:	Location not an angles type*

654:	Invalid procedure overload*

655:	Array index required*

656:	Array index mismatch*

657:	Invalid array index*

658:	Unsupported array access*

659:	Reference frame wrong type*

660:	Reference frame undefined data*

661:	Stack frame does not exist*

662:	Ambiguous Public reference*

663:	Missing module end*

664:	Too many breakpoints*

665:	Duplicate breakpoint*

666:	No instruction at this line*

668:	Objects not allowed for class*

669:	Thread paused in eval*

670:	Unsupported procedure reference*

671:	Missing string*

672:	Object value is Nothing*

673:	Short string*

674:	Invalid property*

675:	Branch not permitted*

676:	Project generated error*

677:	Invalid in shared procedure*

678:	Inconsistent MOVE.TRIGGER mode*

679:	Procedure exception*

680:	Invalid static initializer*

681:	Conveyor must be base RefFrame*

682:	undefined*

683:	Arc cannot transition conveyors*

684:	ZClearance property not set*

685:	XML documents do not match*

686:	No delegate defined*

687:	Object not up-to-date*

688:	No module defined*

689:	XML error*

690:	No XML document*

691:	No XML node*

692:	Undefined XML name*

693:	Invalid XML node type*

694:	XML documents do not match*

695:	Invalid circular reference*

696:	Invalid Const reference*

697:	Invalid exception*

698:	Branch out of Finally block not permitted*

699:	Expression too complex*

700:	Unexpected end of line*

710:	Invalid robot number*

711:	Undefined robot*

712:	Invalid axis number*

713:	Undefined axis*

714:	Invalid motor number*

715:	Undefined motor*

716:	Robot already attached*

717:	Robot not ready to be attached*

718:	Can't detached a moving robot*

719:	No robot attached*

720:	No robot selected*

721:	Illegal during special Cartesian mode*

722:	Joint out-of-range*

723:	Motor out-of-range*

724:	Time out during nulling*

725:	Invalid roll over spec*

726:	Torque control mode incorrect*

727:	Not in position control mode*

728:	Not in velocity control mode*

729:	Timeout sending servo setpoint*

730:	Timeout reading servo status*

731:	Robot not homed*

732:	Invalid homing parameter*

733:	Missed signal during homing*

734:	Encoder index disabled*

735:	Timeout enabling power*

736:	Timeout enabling amp*

737:	Timeout starting commutation*

738:	Hard E-STOP*

739:	Asynchronous error*

740:	Fatal asynchronous error*

741:	Analog input value too small*

742:	Analog input value too big*

743:	Invalid Cartesian value*

744:	Negative overtravel*

745:	Positive overtravel*

746:	Kinematics not installed*

747:	Motors not commutated*

748:	Project generated robot error*

749:	Position too close*

750:	Position too far*

751:	Invalid Base transform*

752:	Can't change robot config*

753:	Asynchronous soft error*

754:	Auto mode disabled*

755:	Soft E-STOP*

756:	Power not enabled*

757:	Virtual MCP in Jog mode*

758:	Hardware MCP in Jog mode*

759:	Timeout on homing DIN*

760:	Illegal during joint motion*

761:	Incorrect Cartesian trajectory mode*

762:	Beyond conveyor limits*

763:	Beyond conveyor limits while tracking*

764:	Can't attach Encoder Only robot*

765:	Cartesian motion not configured*

770:	Invalid Data ID code*

771:	Unknown Data ID code*

772:	Inconsistent duplicate Data ID code*

775:	Uninitialized parameter database*

777:	Must be master node*

779:	Invalid data index*

780:	Database internal consistency error*

781:	Invalid pointer value*

782:	Undefined callback routine*

784:	Invalid parameter array index*

785:	Invalid data file format*

786:	Invalid number of axes or motors*

787:	Invalid signal number*

788:	Undefined signal number*

789:	Output signal required*

790:	Can't open parameter DB file*

791:	Invalid parameter DB file not loaded*

792:	Cannot write value when power on*

793:	Coupled axis must be on same node*

794:	Saving PDB values to flash not allowed*

820:	Data ID cannot be logged*

821:	Invalid when datalogger enabled*

822:	Datalogger not initialized*

823:	No data items defined*

824:	Trigger Data ID must be logged*

825:	Trigger Data IDs on different nodes*

826:	Trigger not allowed for Data ID*

830:	Invalid when CPU Monitor enabled*

831:	No CPU Monitor data available*

840:	Power off requested*

841:	Software Reset: using default settings*

842:	External E-STOP*

843:	Watchdog timer expired*

844:	Power light failure*

845:	Unknown power off request*

846:	E-STOP stuck off*

847:	Trajectory task overrun*

849:	E-STOP timer failed*

850:	Controller overheating*

851:	Auto/Manual switch set to Manual*

852:	Power supply relay stuck*

853:	Power supply shorted*

854:	Power supply overloaded*

855:	No 3-phase power*

856:	Shutdown due to overheating*

857:	CPU overheating*

860:	Cannot get local host name*

861:	Cannot get local host address*

862:	Connection refused*

863:	No connection*

864:	Invalid network address*

865:	Network timeout*

866:	Already connected*

867:	Socket not open*

868:	Connection closed*

869:	Invalid protocol*

870:	Invalid multicast address*

880:	Web interface not enabled*

890:	Modbus exception: n*

891:	Modbus device timeout*

892:	Modbus disable requested*

900:	Servo latency too large*

910:	NULL pointer detected*

911:	Too many arguments*

912:	Too few arguments*

913:	Illegal value*

914:	Servo not initialized*

915:	Servo mode transition failed*

916:	Servo mode locked*

917:	Servo hash table not found*

918:	Servo hash entry collision*

919:	No hash entry found*

920:	Servo hash table full*

921:	Illegal parameter access*

922:	Servo high power failed*

923:	Servo task submission failed*

924:	Cal parameters not set correctly*

925:	Cal position not ready*

926:	Illegal cal seek command*

927:	No axis selected*

930:	Hard envelope error*

932:	Illegal zero index*

933:	Missing zero index*

934:	Motor duty cycle exceeded*

935:	Motor stalled*

936:	Axis over-speed*

937:	Amplifier over-current*

938:	Amplifier over-voltage*

939:	Amplifier under-voltage*

940:	Amplifier fault*

941:	Brake fault*

942:	Excessive dual encoder slippage*

943:	Motor commutation setup failed*

944:	Servo tasks overrun*

945:	Encoder quadrature error*

946:	Precise encoder index error*

947:	Amplifier RMS current exceeded*

948:	Dedicated DINs not config'ed for Hall*

949:	Illegal 6-step number*

950:	Illegal commutation angle*

951:	Encoder fault*

952:	Soft envelope error*

953:	Cannot switch serial encoder mode*

954:	Serial encoder busy*

955:	Illegal encoder command*

956:	Encoder operation error*

957:	Encoder battery low*

958:	Encoder battery down*

959:	Invalid encoder multi-turn data*

960:	Illegal encoder operation mode*

961:	Encoder not supported or mis-matched*

962:	Trajectory extrapolation limit exceeded*

963:	Amplifier fault:	 DC bus stuck*

964:	Encoder data or accel/decel limit error*

965:	Phase offset too large*

966:	Cannot configure to adjust phase offset automatically*

967:	Amplifier hardware failure or invalid configuration*

968:	Encoder position not ready*

969:	Encoder not ready*

970:	Encoder communication error*

971:	Encoder overheated*

972:	Encoder hall sensor error*

973:	General serial bus encoder error*

974:	Amplifier overheating*

975:	Motor overheating*

976:	Earlier encoder error inhibiting power*

981:	Received the command cannot conduct during moving*

982:	Received drive command during error*

983:	Cannot receive commands*

984:	Receive stop command repeatedly*

985:	Data range error*

986:	Format error*

987:	Moving target range is beyond the soft limits*

988:	POINT number error (transmitted POS not POF)*

989:	No selected POINT data*

990:	Cannot conduct the command*

991:	Requested range of POS is too wide*

992:	Command is not existed*

993:	Macro command is already exist*

1012:   InValid Stage

1013:   InValid Hand

1014:   InValid Slot

1051:   No Data

Error Type: 2

1:	Received 'unable to operate' while the robot is working*

2:	Received operation command during error*

3:	Unable to receive any command in this status*

4:	Receive a stop-command again during transmitting stop- command*

5:	Data range error*

6:	Format error*

7:	Motion range went over the software limit*

8:	POINT number error (Transmit POS:	 not POF)*

9:	Appointed POINT data does not exist*

10:	This command is not able to execute*

11:	Required the range of POS is too wide*

12:	Designated command is not exist*

13:	Macro command is defined in duplicate*

99:	Communication Error*

Error Type: 3

97:	Map error*

Error Type: 80

786:	PROJECT_CUSTOM_ERROR

1000:	Invalid Robot Number

1001:	Undefined Robot

1002:	Invalid axis number

1003:	Undefined axis

1004:	Invalid motor number

1005:	Undefined motor

1006:	Robot already attached

1007:	Robot not ready to be attached

1008:	Can't detached a moving robot

1009:	Command is not completed

1010:	No robot selected

1011:	Illegal during special Cartesian mode

1012:	Joint out-of-range

1013:	Motor out-of-range

1014:	Time out during nulling

1015:	Invalid roll over spec

1016:	Torque control mode incorrect

1017:	Not in position control mode

1018:	Not in velocity control mode

1019:	Timeout sending servo setpoint

1020:	Timeout reading servo status

1021:	Robot not homed

1022:	Invalid homing parameter

1023:	Missed signal during homing

1024:	Encoder index disabled

1025:	Timeout enabling power

1026:	Timeout enabling amp

1027:	Timeout starting commutation

1028:	Hard E-STOP

1029:	Asynchronous error

1030:	Fatal asynchronous error

1031:	Analog input value too small

1032:	Analog input value too big

1033:	Invalid Cartesian value

1034:	Negative over travel

1035:	Positive overtravel

1036:	Kinematics not installed

1037:	Motors not commutated

1038:	Project generated robot error

1039:	Position too close

1040:	Position too far

1041:	Invalid Base transform

1042:	Can't change robot config

1043:	Asynchronous soft error

1044:	Auto mode disabled

1045:	Soft E-STOP

1046:	Power not enabled

1047:	Virtual MCP in Jog mode

1048:	Hardware MCP in Jog mode

1049:	Timeout on homing DIN

1050:	Illegal during joint motion

1051:	Incorrect Cartesian trajectory mode

1052:	Beyond conveyor limits

1053:	Beyond conveyor limits while tracking

1054:	Can't attach Encoder Only robot

1055:	Cartesian motion not configured

1522:	Cannot write value when power on

1600:	Power off requested

1601:	Software Reset using default settings

1602:	External E-STOP

1603:	Watchdog timer expired

1604:	Power light failure

1605:	Unknown power off request

1606:	E-STOP stuck off

1607:	Trajectory task overrun

1609:	E-STOP timer failed

1610:	Controller overheating

1611:	Auto/Manual switch set to Manual

1612:	Power supply relay stuck

1613:	Power supply shorted

1614:	Power supply overloaded

1615:	No 3-phase power

1616:	Shutdown due to overheating

1617:	CPU overheating

3000:	NULL pointer detected

3001:	Too many arguments

3002:	Too few arguments

3003:	Illegal value

3004:	Servo not initialized

3005:	Servo mode transition failed

3006:	Servo mode locked

3007:	Servo hash table not found

3008:	Servo hash entry collision

3009:	No hash entry found

3010:	Servo hash table full

3011:	Illegal parameter access

3012:	Servo high power failed

3013:	Servo task submission failed

3014:	Cal parameters not set correctly

3015:	Cal position not ready

3016:	Illegal cal seek command

3017:	No axis selected

3100:	Hard envelope error

3102:	Illegal zero index

3103:	Missing zero index

3104:	Motor duty cycle exceeded

3105:	Motor stalled

3106:	Axis over-speed

3107:	Amplifier over-current

3108:	Amplifier over-voltage

3109:	Amplifier under-voltage

3110:	Amplifier fault

3111:	Brake fault

3112:	Excessive dual encoder slippage

3113:	Motor commutation setup failed

3114:	Servo tasks overrun

3115:	Encoder quadrature error

3116:	Precise encoder index error

3117:	Amplifier RMS current exceeded

3118:	Dedicated DINs not config'ed for Hall

3119:	Illegal 6-step number

3120:	Illegal commutation angle

3121:	Encoder fault

3122:	Soft envelope error

3123:	Can't switch serial encoder mode

3124:	Serial encoder busy

3125:	Illegal encoder command

3126:	Encoder operation error

3127:	Encoder battery low

3128:	Encoder battery down

3129:	Invalid encoder multi-turn data

3130:	Illegal encoder operation mode

3131:	Encoder not supported or mismatched

3132:	Trajectory extrapolation limit exceeded

3133:	Amplifier fault

3134:	Encoder data or accel/decel limit error

3135:	Phase offset too large

3136:	Cannot configure to adjust phase offset automatically

3137:	Amplifier hardware failure or invalid configuration

3138:	Encoder position not ready

3139:	Encoder not ready

3140:	Encoder communication error

3141:	Encoder overheated

3142:	Encoder hall sensor error

3143:	General serial bus encoder error

3144:	Amplifier overheating

3145:	Motor overheating

3147:   Abnormal envelope error

FTR

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Robot to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Robot to return inner status. |
| Get FOUP | 0x03 | P1:Station P2: slot(not use) P3:arm(not use) | 0x62:Completed 0x63: Error Detected. | This command instructs Robot to get a FOUP from specified position |
| Put FOUP | 0x04 | same as up | 0x62:Completed 0x63: Error Detected. | This command instructs Robot to get a FOUP from specified position |
| Ready Get FOUP | 0x05 | same as up | 0x62:Completed 0x63: Error Detected. | This command instructs Robot ready to get a FOUP from specified position |
| Ready Put FOUP | 0x06 | same as up | 0x62:Completed 0x63: Error Detected. | This command instructs Robot ready to get a FOUP from specified position |
| Move to safe | 0x07 | P1:station P2: slot(not use) P3:arm(not use) | 0x62:Completed 0x63: Error Detected. | This command instructs Robot move to safe position |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:: arm has FOUP or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

FTR Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

Error Type: 3

0:	Not Defined

1:	Foup Present

2:	Grip error

3:	Arm error

4:	Foup not present

5:	Foup LP1 NOT Ready

6:	Foup LP6 NOT Ready

7:	LP1 NOT Ready

8:	LP2 NOT Ready  LP

Opener

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Opener to return inner status. |
| Lock Box | 0x03 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to lock box. |
| Unlock Box | 0x04 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to unlock box. |
| OpenBox | 0x05 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to lock box and openbox and mapping. |
| CloseBox | 0x06 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to closebox ,but not unlock |
| GetMap | 0x07 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to return the last map obtained. |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:has foup or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |
| Map Return | 0x73 | P1-P25: mapping status ‘P’ (0x50): Present; ‘E’ (0x45): Empty; ‘C’ (0x43): Crossed; ‘?’ (0x3F): Unknown; ‘D’ (0x44): Double; | The result of a GetMap Data command: |

Opener Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

2:	Permanent macros not loaded

3:	MACRO not loaded*

4:	Unknown Macro Command*

5:	Download command when not in Download mode

6:	Too many permanent Macros

7:	Permanent macro space exceeded

8:	Invalid command while downloading perm

9:	Invalid macro move*

10:	Home to an Illegal Position*

11:	Move failed to reach sensor*

12:	Move start unknown*

13:	Move attempted to an illegal position*

14:	Move failed to detect end*

15:	Position incorrect after move*

16:	Move: Device not Homed*

17:	Multiple Sensors Detected*

18:	Bad NVM Checksum*

19:	Read of Flash Memory Failed*

21:	Invalid Motor*

22:	Invalid Macro Location*

23:	Encoder Count Error*

24:	MACRO in Standby*

26:	Macro Aborted*

27:	Too Much Recursion*

28:	Communication Error*

29:	Too Many Stops for Movement*

30:	Monitor Stop Failure*

32:	Encoder Reset Failure*

33:	Too Many Mapper Returns*

34:	Virtual Stop Failure*

38:	Mapper Failure*

39:	Servo Reset Failure*

40:	NVM Write Failure*

41:	Move Off Sensor Failed*

42:	Software Failure (Rabbit 3000 only)

Error Type: 2

0:	House Air Failure*

1:	House Vacuum Failure*

2:	Slot Moves not Allowed*

3:	Invalid Slot for Move*

4:	Move Restriction Fault*

5:	External Communication Failure*

6:	Invalid Map Move*

7:	Invalid Map Start Location*

8:	Invalid Slot Transitions*

9:	Slot Position Failed*

10:	Unknown POD Type

11:	Too Many POD Returns

12:	Invalid Encoder Move*

13:	No Encoder for Device*

14:	Pellicle Found on Top

15:	Mask Placed Sideways

16:	Position Limit Error

17:	Protrusion Detected

18:	Controller Stopped Motion

19:	Skewed Mask Detected

Error Type: 3

0:	Clamp Cylinder Abnormal

1:	Table Cylinder Abnormal

2:	Latch Cylinder Abnormal

3:	Door Cylinder Abnormal

4:	Mapper Cylinder Abnormal

5:	Vacuum Error

6:	Foup Not Loaded

7:	Foup Not Ready

8: 	Foup Misplaced

9:	Protection Detected

10:	Emergency Stop

11:	N2 Error

12:	Foup Already Loaded

13:	Wafer Protrusion Detected

14:	Z Axis Limit Detected

15:	8" Adapter flag not correct

16:	Adapter sensors not correct

17:	Pod Present - cannot change

18:	Cassette Not Place

Pusher

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Opener to return inner status. |
| Move to HV | 0x03 | P1: Position P2: Not used P3: Comb (0: Dirty, 1: Clean) | 0x62:Completed 0x63: Error Detected. | To Move Elevator under HV and then move Elevator up to Get/ Put position |
| Move to Home | 0x04 | none | 0x62:Completed 0x63: Error Detected. | To Move Elevator down and then move Elevator to Home position. |
| Move Down | 0x05 | none | 0x62:Completed 0x63: Error Detected. | To Move Elevator down. |
| Move Up | 0x06 | none | 0x62:Completed 0x63: Error Detected. | To Move Elevator up |
| Move to Robot Get Position | 0x07 | P1: Not used P2: Not used P3: Comb (0: Dirty, 1: Clean) | 0x62:Completed 0x63: Error Detected. | To Move to getting position of Tool Robot. |
| Rotate | 0x08 | P1: Angle 0： 0 degree 180：180 degree | 0x62:Completed 0x63: Error Detected. | To Rotate Pusher to angle. |
| Mapping | 0x09 | none | 0x73: mapping data 0x63: Error Detected. | To Get Mapping of Pusher. Send Back Message 0x73 Map Response |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:has wafer or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |
| Map Return | 0x73 | P1-P25: mapping status ‘P’ (0x50): Present; ‘E’ (0x45): Empty; ‘C’ (0x43): Crossed; ‘?’ (0x3F): Unknown; ‘D’ (0x44): Double; | The result of a GetMap Data command: |

Pusher Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

HV

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Opener to return inner status. |
| Lock Retainer | 0x03 | none | 0x62:Completed 0x63: Error Detected. | To Lock Retainer to fix Wafers. |
| UnLock Retainer | 0x04 | none | 0x62:Completed 0x63: Error Detected. | To Unlock Retainer to release Wafers. |
| Flip | 0x05 | P1: Angle 0: 0 Degree,； 1: 90 Degree; 2: 180 Degree; 3: 270 Degree | 0x62:Completed 0x63: Error Detected. | To Flip. |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:has wafer or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

HV Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

Aligner

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Opener to return inner status. |
| Align | 0x03 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Aligner to align. |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:has wafer or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

Aligner Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

Loadport

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Opener to return inner status. |
| Lock Box | 0x03 | none | 0x62:Completed 0x63: Error Detected. | Lock box |
| Unlock Box | 0x04 | none | 0x62:Completed 0x63: Error Detected. | Unlock box |
| OpenDoor | 0x05 | none | 0x62:Completed 0x63: Error Detected. | Open door |
| CloseDoor | 0x06 | none | 0x62:Completed 0x63: Error Detected. | Close door |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:has wafer or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

RFID Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

RFID

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Read | 0x03 | P1:channel 1:LP1 2:LP2 3:LP3 4:LP4 | 0x71:RFID data 0x63: Error Detected. | This command instructs RFID Reader to read RFID tag data. |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Tag data response | 0x71 | P1: Channel. P2: Result. ( 0 : OK) P3-n: data | This message is generated upon completion of a Read RFID command |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

RFID Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

FOUP Buffer Clamp

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Software Reset | 0x01 | none | 0x62:Completed 0x63: Error Detected. | This command instructs Opener to reset all axes. |
| Generate Status | 0x02 | none | 0x61:status response 0x63: Error Detected. | This command instructs Opener to return inner status. |
| Rise Up | 0x03 | none | 0x62:Completed 0x63: Error Detected. | To get the Box to Buffer |
| Put Down | 0x04 | none | 0x62:Completed 0x63: Error Detected. | To put the Box to Opener. |
| Release | 0x05 | none | 0x62:Completed 0x63: Error Detected. | To release box to FOUP Robot. |

EFEM  to Host

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Status Response | 0x61 | P1:has wafer or not 1=exist 0=not exist P2: subsystem status. 0x00: Unknown. 0x01: Idle. Not init 0x02: Error. Need to reset. 0x03: Normal. has ready. s P3: subsystem busy status 0x01: busy 0x00: idle | This message is generated when host issues a Generate Status command |
| Operation Completed | 0x62 | P1:command that completed | This message is generated when a command involving extended actions and which has no other completion indicator is completed |
| Error Detected | 0x63 | Parameter 1: Command in Progress Parameter 2: Error Type. Parameter 3: Error Code (High Byte). Parameter 4: Error Code (Low Byte). | This message is generated when Robot detects an error in command execution. The normal response will not be generated |

Error Code Definition

Error Type: 1

0:	Unknown Command

1:	Invalid Command*

- System Command

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| Generate all subsystem status | 0x0000 | none | 0x0000 | Generate all Sub-Systems current error status |
| Generate all Box Stations Present status | 0x0001 | none | 0x0001 | Generate all Box Stations Present status |
| Set Control Mode | 0x0003 | P1: New Control Mode 0: Local Mode 1: Remote Mode | 0x0003 Or NAK | Change control mode |
| Read Control Mode | 0x0004 | none | 0x0003 | Current control mode |
| Pause Operation | 0x0005 | none | 0x0008 |  |
| Resume Operation | 0x0006 | none | 0x0008 |  |
| Are You Alive Request | 0x0031 | none | 0x0017 | Are You Alive Request |
| Get Box ID | 0x0007 | P1: Station | 0x0019 |  |
| Set E84 Access Mode | 0x0008 | P1: Loadport 1:LP1 2:LP2 3:LP3 4:LP4 P2: Mode 0: Manual 1: Auto | 0x0008 |  |
| Get E84 Access Mode | 0x0009 | P1: Loadport 1:LP1 2:LP2 3:LP3 4:LP4 | 0x0020 |  |
| Set Tool Robot Busy | 0x0035 | P1: Sub-System ID of Pusher or Aligner (see Station Definition - Sub-System). P2: Status of Tool Robot (0: Tool Robot Not Busy; 1: Tool Robot Busy) | 0x0008 |  |

- System Response

| Response name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| All Sub-System status response | 0x0000 | P1：subsystem id P2：state PN：subsystem id PN+1：state If state is ERROR ,need to reset it. | state： 0：UNKNOWN 1：IDLE 2：NORMAL 3：ERROR |
| All box present status response | 0x0001 | P1：station id P2：present status PN：station id PN+1：present status |  |
| Current Control Mode response | 0x0003 | P1: 0: local mode; 1: remote mode |  |
| Command completed | 0x0008 | 无 |  |
| Equipment is alive response | 0x0017 | 无 |  |
| Get Box ID response | 0x0019 | P1：id length P2-n:ASCII |  |
| Get E84 Access Mode response | 0x0020 | P1: loadport 1:LP1 2:LP2 3:LP3 4:LP4 P2:mode 0=manual 1=auto |  |

- Defined Command

Host to EFEM

| Command Name | Command ID | Parameter | Response ID | Description |
| --- | --- | --- | --- | --- |
| System Reset | 0x0000 | none | Response: Defined Command Response, ID 0x00 or 0x01 | Reset every Sub-System to its home position and clear all errors. |

- Event

| Event Name | Response ID | Parameter | Description |
| --- | --- | --- | --- |
| Sub-Command completed | 0x0000 | P1： Subsystem P2:Process (0~100) |  |
| Sub-Command error detected | 0x0001 | P1： Subsystem P2:Process (0~100) P3-P6: Error code |  |
| Subsystem Status Changed | 0x0002 | P1： Subsystem |  |
| Box Present | 0x0003 | P1： station |  |
| Box Removed | 0x0004 | P1： station |  |
| Control Mode changed | 0x0005 | P1:current mode 0: local mode; 1: remote mode |  |
| E84 Timeout | 0x0019 | P1：Loadport 1:LP1 2:LP2 3:LP3 4:LP4 P2：Timeout type 1：TP1 2：TP2 3：TP3 4：TP4 5：TP5 6：TP6 |  |

Station Defined

SubSystem/Station (0x00 ~ 0x7F)

0x00:	WTR

0x01: 	FTR

0x02:	LP1

0x03:	LP2

0x04:	LP3

0x05:	LP4

0x06:	Opener1

0x07:	Opener2

0x08:	HV

0x09:	Pusher

0x0A:	Aligner

0x0B:	FOUP Buffer Clamp

0x0C:  Storage 1

0x0D:  Storage 2

0x0E:  Storage 3

0x0F:  Storage 4

0x10:  Storage 5

0x12:  Storage 6

0x13:  Storage 7

0x14:  Storage 8

0x15:  Storage 9

0x16:  Storage 10

0x17: Storage 11

0x18:  Storage 12

0x19:  Storage 13

0x20: Storage 14

- Error Code Definition (4 Bytes)

Byte 1:	Error Source

0x00 ~ 0x7F: the Sub-System.

Byte 2: Error Type (see Sub-System Command/Response)

Byte 3: Error Code (High Byte)

Byte 4: Error Code (Low Byte)

0x80:	System Error

Byte 2, 3: The error code (Byte 2: MByte; Byte 3: LByte)

Byte 4: 0 (not used)

* In Sub-System document, if there is ‘*’ at the last of the error definition like Robot “14: Move failed to detect end*”, it will cause the Sub-System need to be reset when this error happens.

System Error (Type=0)

0x0001		internal driver error

0x0002         		command not support

0x0003		command parameter error

0x0004         		command be abort

0x0005        		data index out of range

0x0006         		command can not lock resource

0x0007         		wafer state exception (unexpected mapping state)

0x0008         		cassette state exception (unexpected cassette present state)

0x0009         		cassette is open exception (unexpected open state)

0x0010         		cassette is close exception (unexpected close state)

0x1000		subsystem is pause now

0x1001		subsystem state is exception,can not execute command

0x1002		subsystem is busy,can not execute command

0x1003		subsystem communication timeout

0x1004		subsystem communication error

0x1005		subsystem communication timeout

0x1006		subsystem communication abort

0x1007		subsystem communication unknown state

0x1008		unexpected door open/close state

0x2000   		robot object state exception (unexpected object state)

0x3000		station whitout cass (expected has cass)

0x3001		station with cass (unexpected has cass)

0x3002		station is conflict

0x3003		box in station is locked

0x3004		box in station can not load or unload

0x3005,		wafer in station can not put or get

0xE000		system runtime logic error

0xE001		system control model  error

0xE002		system busy

0xE003		system alarm ,because of safe

0xE004		system whitout special resource

0xE005		system script error

0xE006		system database error

0xE007		system communication error with host

0xEFFF		system unknown error

EFEM Operation Guide

  - System Start

When EFEM GUI is started, every Sub-System is at the status of ‘Need Reset’. Host should send ‘Defined Command – System Reset’ to reset all Sub-Systems. Then it can start to operate the system.

  - Error Recovery

If there is Sub-System error happens, Host can just reset the Sub-System by sending ‘Sub-System Command – Software Reset’. Or send ‘Defined Command – System Reset’ to reset whole system.

  - Get/Put Carrier

To get/put carrier, Host can send Sub-System Command ‘Robot’ ‘Get Object’/ ‘Put Object’ command directly.